Lab 1

CMPENG 331

Mohammad Abu Shariha

Code:

Src:

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Create Date: 09/07/2025

// Module(s) Name: dff3 and counter

// Project Name: 7 segment display

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

// Mohammad Abu Shariha

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

// Module      : D flip-flop

// Description : At the positive edge of the clock, set the current state based on the clear

// Input(s)    : 3-bit Next State, Clear and Clock

// Output(s)   : 3-bit Current State

module dff3(input [2:0] ns, input clrn, input clk, output reg [2:0] q);

always @ (posedge clk)

begin

if (clrn == 1) begin

q <= ns;

end

else begin

q <= 3'b000;

end

end

endmodule

// Module      : Up/Down Counter

// Description : Combinational circuit that generates the next state and LED control signals based on input u

// Input(s)    : 3-bit Current State, 1-bit u

// Output(s)   : 3-bit Next State, 7 1-bit segments of the LED

module counter(input [2:0] q, input u, output reg [2:0] ns,

output reg a, output reg b, output reg c, output reg d, output reg e, output reg f, output reg g );

always @(*)

begin

if (u == 1) begin

if (q == 3'b101) begin

ns <= 3'b000;

end

else begin

ns <= q + 1;

end

end

else begin

if (q == 3'b000) begin

ns <= 3'b101;

end

else begin

ns <= q - 1;

end

end

case(q)

3'b000: begin

g=1'b1;  f=1'b0;  e=1'b0;  d=1'b0;  c=1'b0;  b=1'b0;  a=1'b0;

end

3'b001: begin

g=1'b1;  f=1'b1;  e=1'b1;  d=1'b1;  c=1'b0;  b=1'b0;  a=1'b1;

end

3'b010: begin

g=1'b0;  f=1'b1;  e=1'b0;  d=1'b0;  c=1'b1  ;b=1'b0;  a=1'b0;

end

3'b011: begin

g=1'b0;  f=1'b1;  e=1'b1;  d=1'b0;  c=1'b0;  b=1'b0;  a=1'b0;

end

3'b100: begin

g=1'b0;  f=1'b0;  e=1'b1;  d=1'b1;  c=1'b0;  b=1'b0;  a=1'b1;

end

3'b101: begin

g=1'b0;  f=1'b0;  e=1'b1;  d=1'b0;  c=1'b0;  b=1'b1;  a=1'b0;

end

endcase

end

endmodule

testbench:

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Engineer: Mohammad Abu Shariha

//

// Create Date: 09/07/2025

// Design Name:

// Module Name: testbench

// Project Name: 7 segment display

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

// Module      : Testbench

// Description : Using the test sequence given generate the output of the 7 segment display by calling dff3 and counter modules

module testbench();

reg clrn_tb;

reg clk_tb;

reg u_tb;

wire [2:0] q_tb;

wire [2:0] ns_tb;

wire a,b,c,d,e,f,g;

dff3 dff3_tb(ns_tb, clrn_tb, clk_tb, q_tb);

counter counter_tb(q_tb, u_tb, ns_tb, a, b, c, d, e, f, g);

initial begin

clrn_tb = 0;

clk_tb = 1;

u_tb = 1;

#1 clrn_tb = 1;

#16 u_tb = 0;

end

always begin

#1;

clk_tb = ~clk_tb;

end

endmodule

Wave Form:

Schematic:

I/O Planning:

Floor Planning: