Mohammad Abu Shariha

CMPENG 331

Lab 4

Section 002

Mmaa7020@psu.edu

Schematics:

I/O planning:

Floor Plan:

Waveform:

Waveform log:

Time resolution is 1 ps

Time | PC | EX/MEM: ALU Out | MEM/WB: Mem Out (wb_mo) | WB Dest Reg

-----------------------------------------------------------------------

Time=0 | PC=00000064 | MEM_ALU=00000000 | WB_MO=00000000 | WB_RN= 0

Time=15000 | PC=00000068 | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

Time=25000 | PC=0000006c | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

Time=35000 | PC=00000070 | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

Time=45000 | PC=00000074 | MEM_ALU=00000004 | WB_MO=a00000aa | WB_RN= 2

Time=55000 | PC=00000078 | MEM_ALU=00000008 | WB_MO=10000011 | WB_RN= 3

Time=65000 | PC=0000007c | MEM_ALU=0000000c | WB_MO=20000022 | WB_RN= 4

Time=75000 | PC=00000080 | MEM_ALU=00000000 | WB_MO=30000033 | WB_RN= 5

Time=85000 | PC=00000084 | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

Time=95000 | PC=00000088 | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

Time=105000 | PC=0000008c | MEM_ALU=00000000 | WB_MO=a00000aa | WB_RN= 0

$finish called at time : 115 ns : File "C:/Users/mma7020/Desktop/Lab4/Lab4.srcs/sources_1/new/testbench.v" Line 74

Verilog design code:

/*

* CMPEN 331 - Lab 4

verilog Design Code for IF, ID, EXE, and MEM Stages

*/

// Top-Level Module: cpu_pipeline

module cpu_pipeline (

input clk,

input rst,

//ignals written into MEM/WB (Output of MEM stage)

output wb_wreg,      // RegWrite

output wb_m2reg,     // MemToReg

output [31:0] wb_mo, // Memory Output Data

output [31:0] wb_alu,// ALU Result (bypass from MEM)

output [4:0] wb_rn,  // Destination Register Number

// Signals output from EX/MEM (Input to MEM stage)

output mem_wreg,

output mem_m2reg,

output mem_wmem,

output [31:0] mem_alu,

output [31:0] mem_r2,

output [4:0] mem_rn

);

//Wires & Interconnects

// IF Stage

wire [31:0] pc_current;

wire [31:0] pc_next;

wire [31:0] inst_if;

// IF/ID Register

wire [31:0] id_inst;

wire [31:0] id_pc4;

//ID

wire [5:0] id_op;

wire [4:0] id_rs;

wire [4:0] id_rt;

wire [4:0] id_rd;

wire [15:0] id_imm16;

wire ctrl_wreg, ctrl_m2reg, ctrl_wmem, ctrl_aluimm, ctrl_regrt;

wire [3:0] ctrl_aluc;

wire [31:0] rf_rd1, rf_rd2;

wire [31:0] imm_ext;

wire [4:0] reg_dest_id;

//ID/EXE Regist-

wire exe_wreg, exe_m2reg, exe_wmem, exe_aluimm;

wire [3:0] exe_aluc;

wire [31:0] exe_rd1, exe_rd2, exe_imm;

wire [4:0] exe_rn;

// EXE Stage

wire [31:0] alu_b_input;

wire [31:0] alu_result_exe;

wire [31:0] mem_data_out;

wire [31:0] wb_data;

assign id_op = id_inst[31:26];

assign id_rs = id_inst[25:21];

assign id_rt = id_inst[20:16];

assign id_rd = id_inst[15:11];

assign id_imm16 = id_inst[15:0];

assign wb_data = (wb_m2reg == 1'b1) ? wb_mo : wb_alu;

// 1.Instruction Fetch (IF) Stage

PC U_PC (

.clk(clk),

.rst(rst),

.next_pc(pc_next),

.current_pc(pc_current)

);

PC_Adder U_PCAdd (

.pc_in(pc_current),

.pc_out(pc_next)

);

InstructionMemory U_IM (

.addr(pc_current),

.inst(inst_if)

);

IFID_Register U_IFID (

.clk(clk),

.rst(rst),

.if_inst(inst_if),

.if_pc4(pc_next),

.id_inst(id_inst),

.id_pc4(id_pc4)

);

//2. Instruction Decode (ID) Stage

ControlUnit U_Control (

.op(id_op),

.wreg(ctrl_wreg),

.m2reg(ctrl_m2reg),

.wmem(ctrl_wmem),

.aluc(ctrl_aluc),

.aluimm(ctrl_aluimm),

.regrt(ctrl_regrt)

);

RegisterFile U_RegFile (

.clk(clk),

.rst(rst),

.rn1(id_rs),

.rn2(id_rt),

.wn(wb_rn),

.wd(wb_data),

.we(wb_wreg),

.rd1(rf_rd1),

.rd2(rf_rd2)

);

SignExtension U_SignExt (

.imm_in(id_imm16),

.imm_out(imm_ext)

);

RegDstMux U_RegDstMux (

.rt(id_rt),

.rd(id_rd),

.sel(ctrl_regrt),

.rn(reg_dest_id)

);

IDEXE_Register U_IDEXE (

.clk(clk),

.rst(rst),

.wreg_in(ctrl_wreg), .m2reg_in(ctrl_m2reg), .wmem_in(ctrl_wmem),

.aluc_in(ctrl_aluc), .aluimm_in(ctrl_aluimm),

.rd1_in(rf_rd1), .rd2_in(rf_rd2), .imm_in(imm_ext), .rn_in(reg_dest_id),

.wreg_out(exe_wreg), .m2reg_out(exe_m2reg), .wmem_out(exe_wmem),

.aluc_out(exe_aluc), .aluimm_out(exe_aluimm),

.rd1_out(exe_rd1), .rd2_out(exe_rd2), .imm_out(exe_imm), .rn_out(exe_rn)

);

// 3Execute (EXE) Stage

ALU_Mux U_ALUMux (

.reg_b(exe_rd2),

.imm(exe_imm),

.sel(exe_aluimm),

.b_out(alu_b_input)

);

ALU U_ALU (

.a(exe_rd1),

.b(alu_b_input),

.aluc(exe_aluc),

.r(alu_result_exe)

);

EXMEM_Register U_EXMEM (

.clk(clk),

.rst(rst),

//nputs

.wreg_in(exe_wreg), .m2reg_in(exe_m2reg), .wmem_in(exe_wmem),

.alu_in(alu_result_exe), .r2_in(exe_rd2), .rn_in(exe_rn),

//Outputs

.wreg_out(mem_wreg), .m2reg_out(mem_m2reg), .wmem_out(mem_wmem),

.alu_out(mem_alu), .r2_out(mem_r2), .rn_out(mem_rn)

);

//4.Memory (MEM) Stage

DataMemory U_DataMem (

.clk(clk),

.addr(mem_alu),

.wdata(mem_r2),

.we(mem_wmem),

.rdata(mem_data_out)

);

MEMWB_Register U_MEMWB (

.clk(clk),

.rst(rst),

.wreg_in(mem_wreg), .m2reg_in(mem_m2reg),

.mo_in(mem_data_out), .alu_in(mem_alu), .rn_in(mem_rn),

.wreg_out(wb_wreg), .m2reg_out(wb_m2reg),

.mo_out(wb_mo), .alu_out(wb_alu), .rn_out(wb_rn)

);

endmodule

// 1. Program Counter

module PC (input clk, rst, input [31:0] next_pc, output reg [31:0] current_pc);

always @(posedge clk, posedge rst) begin

if (rst) current_pc <= 32'd100;

else current_pc <= next_pc;

end

endmodule

// 2.PC Adder

module PC_Adder (input [31:0] pc_in, output [31:0] pc_out);

assign pc_out = pc_in + 32'd4;

endmodule

//3 Instruction Memory

module InstructionMemory (input [31:0] addr, output [31:0] inst);

reg [31:0] rom[0:63];

integer i;

initial begin

for (i=0; i<64; i=i+1) rom[i] = 32'h0;

// 100: lw $2, 00($1)  -> rs=1, rt=2, imm=0  -> 8C220000

// 104: lw $3, 04($1)  -> rs=1, rt=3, imm=4  -> 8C230004

// 108: lw $4, 08($1)  -> rs=1, rt=4, imm=8  -> 8C240008

// 112: lw $5, 12($1)  -> rs=1, rt=5, imm=12 -> 8C25000C

rom[25] = 32'h8C220000;

rom[26] = 32'h8C230004;

rom[27] = 32'h8C240008;

rom[28] = 32'h8C25000C;

end

assign inst = rom[addr[7:2]];

endmodule

// 4.IF/ID Register

module IFID_Register (input clk, rst, input [31:0] if_inst, if_pc4, output reg [31:0] id_inst, id_pc4);

always @(posedge clk, posedge rst) begin

if (rst) begin id_inst <= 0; id_pc4 <= 0; end

else begin id_inst <= if_inst; id_pc4 <= if_pc4; end

end

endmodule

// 5Control Unit

module ControlUnit (

input [5:0] op,

output reg wreg, m2reg, wmem, aluimm, regrt,

output reg [3:0] aluc

);

parameter OP_LW = 6'b100011;

parameter ALU_ADD = 4'b0010;

always @(*) begin

if (op == OP_LW) begin

wreg = 1; m2reg = 1; wmem = 0; aluimm = 1; regrt = 0; aluc = ALU_ADD;

end else begin

wreg = 0; m2reg = 0; wmem = 0; aluimm = 0; regrt = 0; aluc = 4'b0000;

end

end

endmodule

// 6.Register File (Initialize to 0)

module RegisterFile (

input clk, rst,

input [4:0] rn1, rn2, wn,

input [31:0] wd,

input we,

output [31:0] rd1, rd2

);

reg [31:0] regs[0:31];

integer i;

always @(posedge clk, posedge rst) begin

if (rst) for (i=0; i<32; i=i+1) regs[i] <= 0;

else if (we && wn != 0) regs[wn] <= wd;

end

assign rd1 = (rn1 == 0) ? 0 : regs[rn1];

assign rd2 = (rn2 == 0) ? 0 : regs[rn2];

endmodule

// 7.Sign Extension

module SignExtension (input [15:0] imm_in, output [31:0] imm_out);

assign imm_out = {{16{imm_in[15]}}, imm_in};

endmodule

// 8.RegDst Mux

module RegDstMux (input [4:0] rt, rd, input sel, output [4:0] rn);

assign rn = (sel == 1) ? rd : rt;

endmodule

// 9.ID/EXE Register

module IDEXE_Register (

input clk, rst,

input wreg_in, m2reg_in, wmem_in, aluimm_in,

input [3:0] aluc_in,

input [31:0] rd1_in, rd2_in, imm_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out, wmem_out, aluimm_out,

output reg [3:0] aluc_out,

output reg [31:0] rd1_out, rd2_out, imm_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; wmem_out<=0; aluimm_out<=0; aluc_out<=0;

rd1_out<=0; rd2_out<=0; imm_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in; wmem_out<=wmem_in;

aluimm_out<=aluimm_in; aluc_out<=aluc_in;

rd1_out<=rd1_in; rd2_out<=rd2_in; imm_out<=imm_in; rn_out<=rn_in;

end

end

endmodule

// 10. ALU Mu

module ALU_Mux (input [31:0] reg_b, imm, input sel, output [31:0] b_out);

assign b_out = (sel == 1) ? imm : reg_b;

endmodule

//11. ALU

module ALU (input [31:0] a, b, input [3:0] aluc, output reg [31:0] r);

always @(*) begin

case (aluc)

4'b0010: r = a + b; // ADD

default: r = 0;

endcase

end

endmodule

// 12. E/MEM Register

module EXMEM_Register (

input clk, rst,

input wreg_in, m2reg_in, wmem_in,

input [31:0] alu_in, r2_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out, wmem_out,

output reg [31:0] alu_out, r2_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; wmem_out<=0; alu_out<=0; r2_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in; wmem_out<=wmem_in;

alu_out<=alu_in; r2_out<=r2_in; rn_out<=rn_in;

end

end

endmodule

// 13. Data Memory

module DataMemory (

input clk,

input [31:0] addr, wdata,

input we,

output [31:0] rdata

);

reg [31:0] dmem [0:63];

integer i;

initial begin

for (i=0; i<64; i=i+1) dmem[i] = 32'h0;

dmem[0] = 32'hA00000AA;

dmem[1] = 32'h10000011;

dmem[2] = 32'h20000022;

dmem[3] = 32'h30000033;

dmem[4] = 32'h40000044;

dmem[5] = 32'h50000055;

dmem[6] = 32'h60000066;

dmem[7] = 32'h70000077;

dmem[8] = 32'h80000088;

dmem[9] = 32'h90000099;

end

// Synchronous write (Verilog_3.pdf slide 8)

always @(posedge clk) begin

if (we) dmem[addr[7:2]] <= wdata;

end

assign rdata = dmem[addr[7:2]];

endmodule

// 14.MEM/WB Register

module MEMWB_Register (

input clk, rst,

input wreg_in, m2reg_in,

input [31:0] mo_in, alu_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out,

output reg [31:0] mo_out, alu_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; mo_out<=0; alu_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in;

mo_out<=mo_in; alu_out<=alu_in; rn_out<=rn_in;

end

end

endmodule

Test bench code:

/*

* CMPEN 331 - Lab 4 Test Bench

*/

`timescale 1ns/1ps

module testbench;

//Stimulus Signals

reg clk;

reg rst;

wire wb_wreg;

wire wb_m2reg;

wire [31:0] wb_mo;

wire [31:0] wb_alu;

wire [4:0] wb_rn;

wire mem_wreg;

wire mem_m2reg;

wire mem_wmem;

wire [31:0] mem_alu;

wire [31:0] mem_r2;

wire [4:0] mem_rn;

cpu_pipeline UUT (

.clk(clk),

.rst(rst),

.wb_wreg(wb_wreg),

.wb_m2reg(wb_m2reg),

.wb_mo(wb_mo),

.wb_alu(wb_alu),

.wb_rn(wb_rn),

//EX/MEM Outputs

.mem_wreg(mem_wreg),

.mem_m2reg(mem_m2reg),

.mem_wmem(mem_wmem),

.mem_alu(mem_alu),

.mem_r2(mem_r2),

.mem_rn(mem_rn)

);

initial begin

clk = 0;

end

always #5 clk = ~clk;

initial begin

rst = 1'b1;

#15;

rst = 1'b0;

#100;

$finish;

end

initial begin

$display("Time | PC | EX/MEM: ALU Out | MEM/WB: Mem Out (wb_mo) | WB Dest Reg");

$display("-----------------------------------------------------------------------");

$monitor("Time=%0t | PC=%h | MEM_ALU=%h | WB_MO=%h | WB_RN=%d",

$time,

UUT.U_PC.current_pc,

mem_alu,

wb_mo,

wb_rn);

end

endmodule