Lab 5 CMPENG 331

Section 002

Mohammad Abu Shariha

Mma7020@psu.edu

Waveform:

Design schematics:

IO planning:

Floor Planning:

Verilog design code:

// TopLevel Module: cpu_pipeline

module cpu_pipeline (

input clk,

input rst,

output wb_wreg,

output wb_m2reg,

output [31:0] wb_mo,

output [31:0] wb_alu,

output [4:0] wb_rn,

output [4:0] rf_wn,

output [31:0] rf_wd,

output rf_we,

output [31:0] rf_qa,

output [31:0] rf_qb

);

//IF Stag

wire [31:0] pc_current;

wire [31:0] pc_next;

wire [31:0] inst_if;

//IF/ID Register

wire [31:0] id_inst;

wire [31:0] id_pc4;

//ID Stage

wire [5:0] id_op;

wire [4:0] id_rs;

wire [4:0] id_rt;

wire [4:0] id_rd;

wire [15:0] id_imm16;

wire ctrl_wreg, ctrl_m2reg, ctrl_wmem, ctrl_aluimm, ctrl_regrt;

wire [3:0] ctrl_aluc;

wire [31:0] imm_ext;

wire [4:0] reg_dest_id;

// ID/EXE Register

wire exe_wreg, exe_m2reg, exe_wmem, exe_aluimm;

wire [3:0] exe_aluc;

wire [31:0] exe_rd1, exe_rd2, exe_imm;

wire [4:0] exe_rn;

//EXE Stage

wire [31:0] alu_b_input;

wire [31:0] alu_result_exe;

//EX/MEM Register

wire mem_wreg, mem_m2reg, mem_wmem;

wire [31:0] mem_alu, mem_r2;

wire [4:0] mem_rn;

//MEM Stage

wire [31:0] mem_data_out;

//WB Stage

wire [31:0] wb_final_data;

// Decode Assignments

assign id_op = id_inst[31:26];

assign id_rs = id_inst[25:21];

assign id_rt = id_inst[20:16];

assign id_rd = id_inst[15:11];

assign id_imm16 = id_inst[15:0];

// WB Mux Logic (lects between emory Data and ALU Result)

assign wb_final_data = (wb_m2reg == 1'b1) ? wb_mo : wb_alu;

// Inputs to Register File (for reporting)

assign rf_wn = wb_rn;

assign rf_wd = wb_final_data;

assign rf_we = wb_wreg;

//IF Stage

PC U_PC (

.clk(clk), .rst(rst), .next_pc(pc_next), .current_pc(pc_current)

);

PC_Adder U_PCAdd (

.pc_in(pc_current), .pc_out(pc_next)

);

InstructionMemory U_IM (

.addr(pc_current), .inst(inst_if)

);

IFID_Register U_IFID (

.clk(clk), .rst(rst), .if_inst(inst_if), .if_pc4(pc_next),

.id_inst(id_inst), .id_pc4(id_pc4)

);

// 2 ID Stage

ControlUnit U_Control (

.op(id_op), .wreg(ctrl_wreg), .m2reg(ctrl_m2reg), .wmem(ctrl_wmem),

.aluc(ctrl_aluc), .aluimm(ctrl_aluimm), .regrt(ctrl_regrt)

);

RegisterFile U_RegFile (

.clk(clk), .rst(rst),

.rn1(id_rs), .rn2(id_rt),

.wn(rf_wn), .wd(rf_wd), .we(rf_we),

.rd1(rf_qa), .rd2(rf_qb)            // Outputs mapped to module ports

);

SignExtension U_SignExt (

.imm_in(id_imm16), .imm_out(imm_ext)

);

RegDstMux U_RegDstMux (

.rt(id_rt), .rd(id_rd), .sel(ctrl_regrt), .rn(reg_dest_id)

);

IDEXE_Register U_IDEXE (

.clk(clk), .rst(rst),

.wreg_in(ctrl_wreg), .m2reg_in(ctrl_m2reg), .wmem_in(ctrl_wmem),

.aluc_in(ctrl_aluc), .aluimm_in(ctrl_aluimm),

.rd1_in(rf_qa), .rd2_in(rf_qb), .imm_in(imm_ext), .rn_in(reg_dest_id),

.wreg_out(exe_wreg), .m2reg_out(exe_m2reg), .wmem_out(exe_wmem),

.aluc_out(exe_aluc), .aluimm_out(exe_aluimm),

.rd1_out(exe_rd1), .rd2_out(exe_rd2), .imm_out(exe_imm), .rn_out(exe_rn)

);

// EXE Stage

ALU_Mux U_ALUMux (

.reg_b(exe_rd2), .imm(exe_imm), .sel(exe_aluimm), .b_out(alu_b_input)

);

ALU U_ALU (

.a(exe_rd1), .b(alu_b_input), .aluc(exe_aluc), .r(alu_result_exe)

);

EXMEM_Register U_EXMEM (

.clk(clk), .rst(rst),

.wreg_in(exe_wreg), .m2reg_in(exe_m2reg), .wmem_in(exe_wmem),

.alu_in(alu_result_exe), .r2_in(exe_rd2), .rn_in(exe_rn),

.wreg_out(mem_wreg), .m2reg_out(mem_m2reg), .wmem_out(mem_wmem),

.alu_out(mem_alu), .r2_out(mem_r2), .rn_out(mem_rn)

);

// 4. MEM Stage

DataMemory U_DataMem (

.clk(clk), .addr(mem_alu), .wdata(mem_r2), .we(mem_wmem), .rdata(mem_data_out)

);

MEMWB_Register U_MEMWB (

.clk(clk), .rst(rst),

.wreg_in(mem_wreg), .m2reg_in(mem_m2reg),

.mo_in(mem_data_out), .alu_in(mem_alu), .rn_in(mem_rn),

.wreg_out(wb_wreg), .m2reg_out(wb_m2reg),

.mo_out(wb_mo), .alu_out(wb_alu), .rn_out(wb_rn)

);

endmodule

// SubModules

module PC (input clk, rst, input [31:0] next_pc, output reg [31:0] current_pc);

always @(posedge clk, posedge rst) begin

if (rst) current_pc <= 32'd100;

else current_pc <= next_pc;

end

endmodule

module PC_Adder (input [31:0] pc_in, output [31:0] pc_out);

assign pc_out = pc_in + 32'd4;

endmodule

module InstructionMemory (input [31:0] addr, output [31:0] inst);

reg [31:0] rom[0:63];

integer i;

initial begin

for (i=0; i<64; i=i+1) rom[i] = 32'h0;

// 100: lw $2, 00($1)  -> 8C220000

// 104: lw $3, 04($1)  -> 8C230004

// 108: lw $4, 08($1)  -> 8C240008

// 112: lw $5, 12($1)  -> 8C25000C

// 116: add $6, $2, $10 -> rs=2, rt=10(0xA), rd=6, func=32(0x20) -> 004A3020

rom[25] = 32'h8C220000;

rom[26] = 32'h8C230004;

rom[27] = 32'h8C240008;

rom[28] = 32'h8C25000C;

rom[29] = 32'h004A3020;

end

assign inst = rom[addr[7:2]];

endmodule

module IFID_Register (input clk, rst, input [31:0] if_inst, if_pc4, output reg [31:0] id_inst, id_pc4);

always @(posedge clk, posedge rst) begin

if (rst) begin id_inst <= 0; id_pc4 <= 0; end

else begin id_inst <= if_inst; id_pc4 <= if_pc4; end

end

endmodule

module ControlUnit (

input [5:0] op,

output reg wreg, m2reg, wmem, aluimm, regrt,

output reg [3:0] aluc

);

parameter OP_LW = 6'b100011;

parameter OP_R_TYPE = 6'b000000;

parameter ALU_ADD = 4'b0010;

always @(*) begin

if (op == OP_LW) begin

wreg=1; m2reg=1; wmem=0; aluimm=1; regrt=0; aluc=ALU_ADD;

end else if (op == OP_R_TYPE) begin

// RType (add)

wreg=1; m2reg=0; wmem=0; aluimm=0; regrt=1; aluc=ALU_ADD;

end else begin

wreg=0; m2reg=0; wmem=0; aluimm=0; regrt=0; aluc=4'b0000;

end

end

endmodule

module RegisterFile (

input clk, rst,

input [4:0] rn1, rn2, wn,

input [31:0] wd,

input we,

output [31:0] rd1, rd2

);

reg [31:0] regs[0:31];

integer i;

initial begin

for (i=0; i<32; i=i+1) regs[i] = 32'h0;

regs[0] = 32'h00000000;

regs[1] = 32'hA00000AA;

regs[2] = 32'h10000011;

regs[3] = 32'h20000022;

regs[4] = 32'h30000033;

regs[5] = 32'h40000044;

regs[6] = 32'h50000055;

regs[7] = 32'h60000066;

regs[8] = 32'h70000077;

regs[9] = 32'h80000088;

regs[10] = 32'h90000099;

end

always @(posedge clk) begin

if (we && wn != 0) regs[wn] <= wd;

end

assign rd1 = (rn1 == 0) ? 0 : regs[rn1];

assign rd2 = (rn2 == 0) ? 0 : regs[rn2];

endmodule

module SignExtension (input [15:0] imm_in, output [31:0] imm_out);

assign imm_out = {{16{imm_in[15]}}, imm_in};

endmodule

module RegDstMux (input [4:0] rt, rd, input sel, output [4:0] rn);

assign rn = (sel == 1) ? rd : rt;

endmodule

module IDEXE_Register (

input clk, rst,

input wreg_in, m2reg_in, wmem_in, aluimm_in,

input [3:0] aluc_in,

input [31:0] rd1_in, rd2_in, imm_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out, wmem_out, aluimm_out,

output reg [3:0] aluc_out,

output reg [31:0] rd1_out, rd2_out, imm_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; wmem_out<=0; aluimm_out<=0; aluc_out<=0;

rd1_out<=0; rd2_out<=0; imm_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in; wmem_out<=wmem_in;

aluimm_out<=aluimm_in; aluc_out<=aluc_in;

rd1_out<=rd1_in; rd2_out<=rd2_in; imm_out<=imm_in; rn_out<=rn_in;

end

end

endmodule

module ALU_Mux (input [31:0] reg_b, imm, input sel, output [31:0] b_out);

assign b_out = (sel == 1) ? imm : reg_b;

endmodule

module ALU (input [31:0] a, b, input [3:0] aluc, output reg [31:0] r);

always @(*) begin

case (aluc)

4'b0010: r = a + b;

default: r = 0;

endcase

end

endmodule

module EXMEM_Register (

input clk, rst,

input wreg_in, m2reg_in, wmem_in,

input [31:0] alu_in, r2_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out, wmem_out,

output reg [31:0] alu_out, r2_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; wmem_out<=0; alu_out<=0; r2_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in; wmem_out<=wmem_in;

alu_out<=alu_in; r2_out<=r2_in; rn_out<=rn_in;

end

end

endmodule

module DataMemory (input clk, input [31:0] addr, wdata, input we, output [31:0] rdata);

reg [31:0] dmem [0:63];

integer i;

initial begin

for (i=0; i<64; i=i+1) dmem[i] = 32'h0;

dmem[0] = 32'hA00000AA; dmem[1] = 32'h10000011; dmem[2] = 32'h20000022;

dmem[3] = 32'h30000033; dmem[4] = 32'h40000044; dmem[5] = 32'h50000055;

dmem[6] = 32'h60000066; dmem[7] = 32'h70000077; dmem[8] = 32'h80000088;

dmem[9] = 32'h90000099;

end

always @(posedge clk) if (we) dmem[addr[7:2]] <= wdata;

assign rdata = dmem[addr[7:2]];

endmodule

module MEMWB_Register (

input clk, rst,

input wreg_in, m2reg_in,

input [31:0] mo_in, alu_in,

input [4:0] rn_in,

output reg wreg_out, m2reg_out,

output reg [31:0] mo_out, alu_out,

output reg [4:0] rn_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

wreg_out<=0; m2reg_out<=0; mo_out<=0; alu_out<=0; rn_out<=0;

end else begin

wreg_out<=wreg_in; m2reg_out<=m2reg_in;

mo_out<=mo_in; alu_out<=alu_in; rn_out<=rn_in;

end

end

endmodule

Test bench code:

`timescale 1ns/1ps

module testbench;

reg clk;

reg rst;

//Monitored ignals

//rom MEM/WB Register

wire wb_wreg;

wire wb_m2reg;

wire [31:0] wb_mo; // Memory Output

wire [31:0] wb_alu;// ALU Output

wire [4:0] wb_rn;  // Des Register

wire [4:0] rf_wn;

wire [31:0] rf_wd;

wire rf_we;

wire [31:0] rf_qa;

wire [31:0] rf_qb;

//-UUT

cpu_pipeline UUT (

.clk(clk),

.rst(rst),

.wb_wreg(wb_wreg),

.wb_m2reg(wb_m2reg),

.wb_mo(wb_mo),

.wb_alu(wb_alu),

.wb_rn(wb_rn),

.rf_wn(rf_wn),

.rf_wd(rf_wd),

.rf_we(rf_we),

.rf_qa(rf_qa),

.rf_qb(rf_qb)

);

initial begin

clk = 0;

end

always #5 clk = ~clk;

initial begin

rst = 1'b1;

#15;

rst = 1'b0;

#150;

$finish;

end

//onitor

initial begin

$display("Time | PC | WB_MO (Load Result) | WB_ALU (Add Result) | WB_RN | RF_QA | RF_QB");

$display("-----------------------------------------------------------------------------------");

$monitor("Time=%0t | PC=%h | WB_MO=%h | WB_ALU=%h | RN=%d | QA=%h | QB=%h",

$time,

UUT.U_PC.current_pc,

wb_mo,

wb_alu,

wb_rn,

rf_qa,

rf_qb);

end

endmodule