CMPENG 331 Lab 3

Mohammad Abu Shariha

Section 002

Mma7020@psu.edu

Waveforms:

Design schematics:

I/O Planning:

Floor Planning:

Verilog Design code:

/*

* CMPEN 331 Lab 3

*/

// toplevel Module: cpu_if_id

module cpu_if_id (

input clk,

input rst,

// Control Signals

output exe_wreg,

output exe_m2reg,

output exe_wmem,

output [3:0] exe_aluc,

output exe_aluimm,

// Data

output [31:0] exe_rd1,

output [31:0] exe_rd2,

output [31:0] exe_imm,

output [4:0] exe_regrt,

output [31:0] exe_pc_plus_4

);

//IF Stage Wire

wire [31:0] pc_current;

wire [31:0] pc_next; //PC + 4

wire [31:0] inst_if;

//IF/ID Register Wires

wire [31:0] inst_id;

wire [31:0] pc_plus_4_id;

// ID Stage Wires

wire [5:0] id_op;

wire [4:0] id_rs;

wire [4:0] id_rt;

wire [4:0] id_rd;

wire [5:0] id_func;

wire [15:0] id_imm16;

//Control Unit outputs

wire ctrl_wreg;

wire ctrl_m2reg;

wire ctrl_wmem;

wire [3:0] ctrl_aluc;

wire ctrl_aluimm;

wire ctrl_RegDst;

wire [31:0] rf_rd1;

wire [31:0] rf_rd2;

// SignExtension output

wire [31:0] imm_ext_32;

// RegDst Mux output

wire [4:0] regrt_id;

wire [4:0] wb_wa = 5'b0;

wire [31:0] wb_wd = 32'b0;

wire wb_we = 1'b0;

// Slicing instruction in ID sta

assign id_op = inst_id[31:26];

assign id_rs = inst_id[25:21];

assign id_rt = inst_id[20:16];

assign id_rd = inst_id[15:11];

assign id_func = inst_id[5:0];

assign id_imm16 = inst_id[15:0];

// IF Stage-

// 1. (PC)

PC U_PC (

.clk(clk),

.rst(rst),

.next_pc(pc_next),

.current_pc(pc_current)

);

// 2.(PC + 4)

PC_Adder U_ADD (

.pc_in(pc_current),

.pc_out(pc_next)

);

// 3. IM

InstructionMemory U_IM (

.addr(pc_current),

.inst(inst_if)

);

// 4. IF/ID

IFID_Register U_IFID (

.clk(clk),

.rst(rst),

.if_inst_in(inst_if),

.if_pc_plus_4_in(pc_next),

.id_inst_out(inst_id),

.id_pc_plus_4_out(pc_plus_4_id)

);

// ID Stage

// 5. Control Unit

ControlUnit U_CU (

.op(id_op),

.wreg(ctrl_wreg),

.m2reg(ctrl_m2reg),

.wmem(ctrl_wmem),

.aluc(ctrl_aluc),

.aluimm(ctrl_aluimm),

.RegDst(ctrl_RegDst)

);

// 6. Reg File

RegisterFile U_RF (

.clk(clk),

.rst(rst),

.rr1(id_rs),

.rr2(id_rt),

.wa(wb_wa),

.we(wb_we),

.wd(wb_wd),

.rd1(rf_rd1),

.rd2(rf_rd2)

);

// 7.Sign Exten

SignExtension U_SE (

.imm_in(id_imm16),

.imm_out(imm_ext_32)

);

// 8. for 'regrt' signal)

RegDstMux U_MUX (

.in_rt(id_rt),

.in_rd(id_rd),

.sel(ctrl_RegDst),

.out_regrt(regrt_id)

);

// 9. ID/EXE Pipeline Register

IDEXE_Register U_IDEXE (

.clk(clk),

.rst(rst),

//Inputs from ID Stage

.in_wreg(ctrl_wreg),

.in_m2reg(ctrl_m2reg),

.in_wmem(ctrl_wmem),

.in_aluc(ctrl_aluc),

.in_aluimm(ctrl_aluimm),

.in_rd1(rf_rd1),

.in_rd2(rf_rd2),

.in_imm(imm_ext_32),

.in_regrt(regrt_id),

.in_pc_plus_4(pc_plus_4_id),

// EXE Stage

.out_wreg(exe_wreg),

.out_m2reg(exe_m2reg),

.out_wmem(exe_wmem),

.out_aluc(exe_aluc),

.out_aluimm(exe_aluimm),

.out_rd1(exe_rd1),

.out_rd2(exe_rd2),

.out_imm(exe_imm),

.out_regrt(exe_regrt),

.out_pc_plus_4(exe_pc_plus_4)

);

endmodule

// 1 Program Counter

module PC (

input clk,

input rst,

input [31:0] next_pc,

output reg [31:0] current_pc

);

always @(posedge clk, posedge rst) begin

if (rst)

current_pc <= 32'd100;

else

current_pc <= next_pc;

end

endmodule

// 2. PC Adder

module PC_Adder (

input [31:0] pc_in,

output [31:0] pc_out

);

assign pc_out = pc_in + 32'd4;

endmodule

// 3. Instruction Memory

module InstructionMemory (

input [31:0] addr,

output [31:0] inst

);

reg [31:0] rom[0:63];

integer i;

initial begin

//Initialize all to NOP (sll $0, $0, 0)

for (i = 0; i < 64; i = i + 1) begin

rom[i] = 32'h00000000;

end

// Load the lab instructions

rom[25] = 32'h8C220000; // Address 100

rom[26] = 32'h8C230004; // Address 104

end

// Asynchronous read ( get from Vverilog3slide 10/13)

assign inst = rom[addr[7:2]];

endmodule

// 4.IF/ID Pipeline Register

module IFID_Register (

input clk,

input rst,

input [31:0] if_inst_in,

input [31:0] if_pc_plus_4_in,

output reg [31:0] id_inst_out,

output reg [31:0] id_pc_plus_4_out

);

always @(posedge clk, posedge rst) begin

if (rst) begin

id_inst_out <= 32'h00000000;

id_pc_plus_4_out <= 32'h00000000;

end else begin

id_inst_out <= if_inst_in;

id_pc_plus_4_out <= if_pc_plus_4_in;

end

end

endmodule

// 5. Control Unit

module ControlUnit (

input [5:0] op,

output reg wreg,

output reg m2reg,

output reg wmem,

output reg [3:0] aluc,

output reg aluimm,

output reg RegDst

);

// Verilog2 75

parameter OP_LW = 6'b100011;

parameter ALU_ADD = 4'b0010;

// Verilog_1 95 and Verilo2 p25

always @(*) begin

if (op == OP_LW) begin

wreg   = 1'b1;

m2reg  = 1'b1;

wmem   = 1'b0; // Not writing to memory

aluc   = ALU_ADD;

aluimm = 1'b1;

RegDst = 1'b0;

end

else begin

wreg   = 1'b0;

m2reg  = 1'b0;

wmem   = 1'b0;

aluc   = 4'b0000;

aluimm = 1'b0;

RegDst = 1'b0;

end

end

endmodule

// 6. Register File

module RegisterFile (

input clk,

input rst,

input [4:0] rr1,

input [4:0] rr2,

input [4:0] wa,  // Write register address

input we,  // Write enable

input [31:0] wd,

output [31:0] rd1, //Read data 1

output [31:0] rd2  //ead data 2

);

reg [31:0] registers[0:31];

//  Verilog3, slide 13)

integer i;

always @(posedge clk, posedge rst) begin

if (rst) begin

for (i = 0; i < 32; i = i + 1) begin

registers[i] <= 32'h00000000;

end

end

else if (we && (wa != 5'b0)) begin

registers[wa] <= wd;

end

end

assign rd1 = (rr1 == 5'b0) ? 32'b0 : registers[rr1];

assign rd2 = (rr2 == 5'b0) ? 32'b0 : registers[rr2];

endmodule

// 7. Sign Extension

module SignExtension (

input [15:0] imm_in,  // 16 immediate

output [31:0] imm_out // 32t sign-extended immediate

);

// erilog1, slide 60)

assign imm_out = {{16{imm_in[15]}}, imm_in};

endmodule

// 8. RegDst Mux (for 'regrt')

module RegDstMux (

input [4:0] in_rt,

input [4:0] in_rd, // From inst[15:11]

input sel,

output [4:0] out_regrt

);

// Verilog2f, slide 8)

assign out_regrt = (sel == 1'b0) ? in_rt : in_rd;

endmodule

// 9. ID/EXE Pipeline Register

module IDEXE_Register (

input clk,

input rst,

input in_wreg,

input in_m2reg,

input in_wmem,

input [3:0] in_aluc,

input in_aluimm,

input [31:0] in_rd1,

input [31:0] in_rd2,

input [31:0] in_imm,

input [4:0] in_regrt,

input [31:0] in_pc_plus_4,

// Outputs to EXE Stage

output reg out_wreg,

output reg out_m2reg,

output reg out_wmem,

output reg [3:0] out_aluc,

output reg out_aluimm,

output reg [31:0] out_rd1,

output reg [31:0] out_rd2,

output reg [31:0] out_imm,

output reg [4:0] out_regrt,

output reg [31:0] out_pc_plus_4

);

// On reset, clear all values (flushes the pipeline)

always @(posedge clk, posedge rst) begin

if (rst) begin

out_wreg <= 1'b0;

out_m2reg <= 1'b0;

out_wmem <= 1'b0;

out_aluc <= 4'b0;

out_aluimm <= 1'b0;

out_rd1 <= 32'b0;

out_rd2 <= 32'b0;

out_imm <= 32'b0;

out_regrt <= 5'b0;

out_pc_plus_4 <= 32'b0;

end else begin

out_wreg <= in_wreg;

out_m2reg <= in_m2reg;

out_wmem <= in_wmem;

out_aluc <= in_aluc;

out_aluimm <= in_aluimm;

out_rd1 <= in_rd1;

out_rd2 <= in_rd2;

out_imm <= in_imm;

out_regrt <= in_regrt;

out_pc_plus_4 <= in_pc_plus_4;

end

end

endmodule

Test bench code:

/*

* CMPEN 331 Lab 3 test bench

*/

`timescale 1ns/1ps

module testbench;

reg clk;

reg rst;

wire exe_wreg;

wire exe_m2reg;

wire exe_wmem;

wire [3:0] exe_aluc;

wire exe_aluimm;

wire [31:0] exe_rd1;

wire [31:0] exe_rd2;

wire [31:0] exe_imm;

wire [4:0] exe_regrt;

wire [31:0] exe_pc_plus_4;

cpu_if_id UUT (

.clk(clk),

.rst(rst),

.exe_wreg(exe_wreg),

.exe_m2reg(exe_m2reg),

.exe_wmem(exe_wmem),

.exe_aluc(exe_aluc),

.exe_aluimm(exe_aluimm),

.exe_rd1(exe_rd1),

.exe_rd2(exe_rd2),

.exe_imm(exe_imm),

.exe_regrt(exe_regrt),

.exe_pc_plus_4(exe_pc_plus_4)

);

//Generate Clock

initial begin

clk = 0;

end

always #5 clk = ~clk;

//  (Reset)

initial begin

rst = 1'b1;

#15;

rst = 1'b0;

#50;

$finish;

end

initial begin

$monitor("Time=%0t ns\n\

\"  --- IF Stage ---\n\

\"  PC_current:       %d (0x%h)\n\

\"  --- IF/ID Register (Internal) ---\n\

\"  IF/ID_inst_out:   0x%h\n\

\"  IF/ID_pc_plus_4:  %d (0x%h)\n\

\"  --- ID/EXE Register (Outputs) ---\n\

\"  ID/EXE_wreg:      %b\n\

\"  ID/EXE_m2reg:     %b\n\

\"  ID/EXE_wmem:      %b\n\

\"  ID/EXE_aluimm:    %b\n\

\"  ID/EXE_aluc:      0x%h\n\

\"  ID/EXE_rd1:       0x%h\n\

\"  ID/EXE_rd2:       0x%h\n\

\"  ID/EXE_imm:       0x%h\n\

\"  ID/EXE_regrt:     %d\n\

\"  ID/EXE_pc_plus_4: %d (0x%h)\n\

\"=======================================\n",

$time,

UUT.U_PC.current_pc, UUT.U_PC.current_pc,

UUT.U_IFID.id_inst_out,

UUT.U_IFID.id_pc_plus_4_out, UUT.U_IFID.id_pc_plus_4_out,

exe_wreg,

exe_m2reg,

exe_wmem,

exe_aluimm,

exe_aluc,

exe_rd1,

exe_rd2,

exe_imm,

exe_regrt,

exe_pc_plus_4, exe_pc_plus_4

);

end

endmodule